424 Meeting 8

Hayes Griffin, Henry Meiring, Frank Jones, Ernesto de Losada,Nigel Malaba

Power Test -- Shaft Failure

- Drilled and tapped hole through coupling and shaft on generator side to bolt the two components together

- Fully functional pre-test; after startup sequence and a few seconds of operation the shaft from the gas engine side became unscrewed and flew off

Shaft Redesign

- Ordered materials to create new shafts

- New shafts designed to have: larger diameter, bolted connection (instead of threaded), solid cross-section, and a longer keyway

- Proceed with Lovejoy coupling

Mounting Preliminary Design

- Preliminary design combines aluminum bars (torque resistance) with carbon fiber tubes (weight) to mount three MG-sets parallel to one another in the approximately 10.5” space between the “6-pointed star” top level and the new base

Drone Body and Frame Design

- Original base design (lacking in structural strength):

  - Isometric:				Side View:

- Complete redesign of hexagonal base:

  - Isometric: 					  Top View:

  - Side View:

  - Updates:

    - The joints are currently being printed and will be ready for assembly

    - The sides of the base will be made of carbon fiber piping, the same tubing used in the main upper body of the drone

- Electronics baseplate

  - Three carbon fiber rods will come together at the center of the hexagon to support a baseplate that will be 3D printed

  - The plate will have a grid of holes in order to allow for the electronics to be securely and evenly attached

  - Constraints: High stability and minimum area required

  - Objectives: Minimize area (weight) while maintaining stability for flight systems